Demo 33: Monitor and control ESP32 via IBM Bluemix Watson IoT Platform

1. Introduction
2. Hardware
3. Bluemix setup steps and ESP32 software
4. How to monitor and control ESP32
    4.1 Node-Red
    4.2 Using own developed Python application 
5. Using graph to display data
6. Using Alert Notification
7. Message Hub 

1. Introduction
In this demo, I will show you how to monitor and control ESP32 via IBM Bluemix Watson IoT Platform.
IBM Bluemix is a cloud platform that supports many products and services such as: Compute Infrastructure, Compute Services, Storage, Mobile, Wason, Application services, Data and analytics, Internet of Things, ...
In this demo, we just focus on Internet of Things service of Bluemix.
In order to use Bluemix, you need to register an account. I registered a free account and it is available in 30 trial days.
You can sign up here. It is easy just follow the steps in registration form.
Here is the model of our demo: ESP32 communicate with Node-RED and Python app via IBM Bluemix Watson IoT Platform.

Figure: ESP32 communicate with Node-RED, Python app via Bluemix

2. Hardware
You need a LED or relay as in Demo 1: Blinky
3. Bluemix setup steps and ESP32 software
Here is the Menu of Bluemix services. It is in top-left corner.

But before using IoT service we need to create a Space (where you deploy your IoT application) for it. It is under Manage - Account - Organizations tabs.

From Menu choose IoT service

Choose Create IoT Service and Internet of Thins Platform

Edit fields as your expectation and choose Create ... Done

From Menu choose Dashboard

Choose IoT service which you created and choose Launch

Now you see IBM Watson IoT Platform dashboard with Menu

Choose Devices and Add Device

Before adding new device, It will recommend you to add Device Type and related device information first.

Here you can fill it with expected information or leave them empty (modify them later).

After creating Device Type, you can add new device.

Here, Device ID is MAC address of ESP32. You can use the code below to get MAC address:

uint64_t chipid; void setup() { Serial.begin(115200); } void loop() { chipid=ESP.getEfuseMac();//The chip ID is essentially its MAC address(length: 6 bytes). Serial.printf("ESP32 Chip ID = %04X",(uint16_t)(chipid>>32));//print High 2 bytes Serial.printf("%08X\n",(uint32_t)chipid);//print Low 4bytes. delay(3000); }

And let authentication token is Auto-generated. Finally, you have:

Remember to save this information for future use.
Note:  TLS with Token Authentication is enabled by default. So if your device use TLS you can ignore it. In my demo using ESP32, I do not use TLS so I have to disable it to avoid Authentication error. In order to disable it just chose Security from Menu.

Choose Connection Security and then TLS Optional.

Now choose Devices from Menu, you will see a list of devices with Disconnected sign. It is time to connect our devices first time.

In order to connect to IBM Watson™ IoT Platform, we will use MQTT protocol (Demo 14). You can refer document to understand the procedure to connect to it. The Arduino ESP32 code will do the jobs:
- Connect to cloud.
- Publish a temperature message (generated by using random function). If you have DHT11/DHT22 you can use Demo 3.
- Subscribe the ON/OFF message to control LED.

#include <WiFi.h> #include <WiFiClient.h> #include <PubSubClient.h> //-------- your wifi ----------- const char* ssid = "dd-wrt"; const char* password = "0000000000"; #define ORG "910d3w" #define DEVICE_TYPE "ESP32_IoTSharing" #define DEVICE_ID "A09E02A4AE30" #define TOKEN "b(N4qP3@Lx!ydaoiEM" //-------- Bluemix information to build up MQTT message ------- char server[] = ORG ".messaging.internetofthings.ibmcloud.com"; char pubTopic[] = "iot-2/evt/status/fmt/json"; char subTopic[] = "iot-2/cmd/test/fmt/String"; char authMethod[] = "use-token-auth"; char token[] = TOKEN; char clientId[] = "d:" ORG ":" DEVICE_TYPE ":" DEVICE_ID; WiFiClient wifiClient; PubSubClient client(server, 1883, NULL, wifiClient); const char led = 4; void receivedCallback(char* pubTopic, byte* payload, unsigned int length) { Serial.print("Message received: "); Serial.println(pubTopic); Serial.print("payload: "); for (int i = 0; i < length; i++) { Serial.print((char)payload[i]); } Serial.println(); /* we got '1' -> on */ if ((char)payload[0] == '1') { digitalWrite(led, HIGH); } else { /* we got '0' -> on */ digitalWrite(led, LOW); } } void setup() { Serial.begin(115200); Serial.println(); pinMode(led, OUTPUT); Serial.print("Connecting to "); Serial.print(ssid); WiFi.begin(ssid, password); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println(""); Serial.print("WiFi connected, IP address: "); Serial.println(WiFi.localIP()); if (!client.connected()) { Serial.print("Reconnecting client to "); Serial.println(server); while (!client.connect(clientId, authMethod, token)) { Serial.print("."); delay(500); } client.setCallback(receivedCallback); if (client.subscribe(subTopic)) { Serial.println("subscribe to cmd OK"); } else { Serial.println("subscribe to cmd FAILED"); } Serial.println("Bluemix connected"); } } long lastMsg = 0; long temperature = 0; void loop() { client.loop(); long now = millis(); if (now - lastMsg > 3000) { lastMsg = now; temperature = random(0, 40); String payload = "{\"d\":{\"Name\":\"" DEVICE_ID "\""; payload += ",\"temperature\":"; payload += temperature; payload += "}}"; Serial.print("Sending payload: "); Serial.println(payload); if (client.publish(pubTopic, (char*) payload.c_str())) { Serial.println("Publish ok"); } else { Serial.println("Publish failed"); } } }

4. How to monitor and control ESP32
We went through steps to setup Bluemix IoT Platform and ESP32. Now we will learn how to develop or setup applications to monitor and control ESP32 Iot device. There are some ways to do that. In this demo, I will use 2 ways: Node-Red (you can refer Demo 8 to know more about Node-RED) and own developed Python application.
4.1 Node-Red
We will use Bluemix Node-RED Starter.
From Menu choose Clound Foundry Apps and then Create Clound Foundry apps. From Filter box search "Node-RED Starter". Choose "Node-RED Starter" and fill expactation information.

Click Create

Waiting until service was deployed ...

Figure: Node-RED service was deployed successfully

Next, from Menu choose Dashboard and you can see "Node-RED" is running. And you can access Node-RED flow editor online by going to the link in Route column (it is "iotsharing.mybluemix.net")

Now, we need to bind our Node-RED starter with our created Bluemix IoT Platform service. From dashboard and Cloud Foundry Apps table, click the Node-RED row (picture above).

Choose Connect existing and choose the Bluemix IoT Platform service which we created in Section 3 Bluemix setup steps and ESP32 software (the result is similar to picture above).

Figure: Bind Node-RED with Bluemix IoT Platform

4.1.1 Let 's play with Node-RED 
We go to the link in Route column ("iotsharing.mybluemix.net").

And choose Go to your Node-RED flow editor (you may need to wait a little time to initialize Node-RED flow editor if you see 404 error).
Finally, it is done.

Node-RED Starter supports 2 nodes named IBM IoT in/out to connect to Bluemix IoT Platform easily. I created the flow as picture above and Export that flow to JSON format. So you just Import the JSON content into your Flow and change Device Information accordingly to the device which you created in Section 3 Bluemix setup steps and ESP32 software. In my Flow there are 2 blocks: 1 subscribe block (upper block; using IBM IoT in) to subscribe the event from ESP32 IoT device and bring to Debug block through Debug tab (Top-Right corner); 1 publish block (lower block; using IBM IoT out) to publish command ON/OFF LED to ESP32 IoT device.

Figure: Import JSON content to Flow

Figure: Change Device Type and Device Id to yours

4.1.2 Result

Figure: Node-RED receive event from ESP32 IoT in debug tab

4.2 Using own developed Python application
4.2.1 Setup 
We will use PAHO MQTT Python (refer Demo 14) for our application.
First, we need to generate API Key for our application.
From IBM Watson IoT Platform dashboard chose APPS:

Then choose Generate API Key

Remember to save this information for future use.

Here is the full Python code using Paho MQTT

from random import randint import thread import sys try: import paho.mqtt.client as mqtt except ImportError: import os import inspect cmd_subfolder = os.path.realpath(os.path.abspath(os.path.join(os.path.split(inspect.getfile( inspect.currentframe() ))[0],"../src"))) if cmd_subfolder not in sys.path: sys.path.insert(0, cmd_subfolder) import paho.mqtt.client as mqtt ORG = "910d3w" DEVICE_TYPE = "ESP32_IoTSharing" APP_ID = "iotsharingdotcom" #choose any string TOKEN = "86a2mx(YZaYo0*&)B6" DEVICE_ID = "A09E02A4AE30" server = ORG + ".messaging.internetofthings.ibmcloud.com"; pubTopic = "iot-2/type/" + DEVICE_TYPE + "/id/" + DEVICE_ID + "/cmd/test/fmt/String"; subTopic = "iot-2/type/+/id/+/evt/+/fmt/+"; authMethod = "a-910d3w-io5gxbfau6"; token = TOKEN; clientId = "a:" + ORG + ":" + APP_ID; def on_connect(mqttc, obj, flags, rc): print("rc: "+str(rc)) def on_message(mqttc, obj, msg): print(msg.topic+" "+str(msg.qos)+" "+str(msg.payload)) def on_publish(mqttc, obj, mid): print("mid: "+str(mid)) def on_subscribe(mqttc, obj, mid, granted_qos): print("Subscribed: "+str(mid)+" "+str(granted_qos)) def on_log(mqttc, obj, level, string): print(string) mqttc = mqtt.Client(client_id=clientId) mqttc.on_message = on_message mqttc.on_connect = on_connect mqttc.on_publish = on_publish mqttc.on_subscribe = on_subscribe mqttc.username_pw_set(authMethod, token) mqttc.connect(server, 1883, 60) mqttc.subscribe(subTopic, 0) def ledControl( threadName, delay): while True: val = raw_input('Enter on or off ') if(val == "on"): mqttc.publish(pubTopic, "1") else: mqttc.publish(pubTopic, "0") try: thread.start_new_thread( ledControl, ("ledControl", 0, ) ) except: print "Error: unable to start thread" mqttc.loop_forever()